INSTRUCTIONS FOR RUNNING THE PROGRAM FLEX.EXE

PROGRAM WRITTEN BY:

ALAN ADLER

81 ENCINA AVENUE

PALO ALTO, CA 94301

PATENTS ARE PENDING ON FLEX TECHNOLOGY.  THIS PROGRAM AND

FLEX TECHNOLOGY MAY BE FREELY USED AND DISTRIBUTED FOR

NON-COMMERCIAL PURPOSES.  COMMERCIAL USERS SHOULD CONTACT

THE ABOVE.

STARTING THE PROGRAM:

FROM DOS TYPE C:\PATH\FLEX <RETURN>

FROM WINDOWS YOU CAN DO ANY ONE OF THE FOLLOWING:

OPEN A DOS WINDOW AND TYPE C:\PATH\FLEX <RETURN>

DOUBLE CLICK ON FLEX.EXE IN WINDOWS EXPLORER

CLICK ON "START", THEN "RUN" AND TYPE C:\PATH\FLEX

(YOU CAN ALSO CREATE A SHORTCUT TO FLEX.EXE)

DATA INPUT:

THE OPENING SCREEN OFFERS OPTIONS TO ENTER DATA OR LOAD DATA

FROM A FILE.  FOR YOUR FIRST TIME, CHOOSE "L" FOR LOAD AND

ENTER THE FILENAME 86 WHICH IS A 8" f6 SAMPLE FILE.

USE THE CURSOR KEYS (NOT A MOUSE) TO MOVE AROUND THE ENTRY

SCREEN AND FILL IN THE NINE LINES: DIAMETER (INCHES), f NUMBER,

SECONDARY DIAMETER ETC.   SOME LINES OFFER THE ENTRY OPTION 0

(ZERO).  FOR EXAMPLE 0 SECONDARY DIAMETER TELLS THE PROGRAM

TO CALCULATE A GOOD SECONDARY DIAMETER FOR THAT MIRROR.

DIMENSIONS ARE IN INCHES.

THE BEST THICKNESS IS ABOUT ONE TENTH OF THE MIRROR DIAMETER.

FOR EXAMPLE, FOR AN 8" MIRROR ENTER 0.8" OR 0.75".

BETA1 IS THE RATIO OF THE ANNULAR PULLER INNER DIAMETER TO

THE MIRROR DIAMETER.

BETA2 IS THE RATIO OF THE ANNULAR PULLER OUTER DIAMETER TO

THE MIRROR DIAMETER.

YOU HAVE THE FOLLOWING OPTIONS WITH BETA1 AND/OR BETA2:

A. ENTER A VALUE (I.E. 0.20).

B. ENTER 1 AND THE PROGRAM WILL CHOOSE THE BETA BASED ON

AN APPROXIMATION RULE.  THE RULE WILL PROPERLY CONSIDER

THE BACK SHAPE SPEC.

THESE FIRST TWO OPTIONS RUN VERY FAST.

C. IF YOU ENTER 0 FOR EITHER OR BOTH BETAS, THE PROGRAM

WILL SEARCH METHODICALLY FOR THE BETA (OR BETAS) THAT

YIELD THE BEST IMAGE.  THIS OPTION TAKES A BIT LONGER.

IN MOST CASES IT WILL FIND THE OPTIMUM, BUT WITH UNUSUAL

GEOMETRIES OPTION "A" MAY BE NECESSARY.

FOR CENTER-PULL SET BETA1=.01 AND BETA2=.05 to 0.10

FOR VACUUM PULL ON THE ENTIRE BACK, SET BETA1=.01 AND

BETA2=1-1.5/DIAMETER.  THIS FORMULA ALLOWS FOR A 0.5" WIDE

PERIMETER SUPPORT AND A 0.25" RADIAL GAP BETWEEN THE INSIDE

EDGE OF THE PERIMETER SUPPORT AND THE OUTSIDE EDGE OF THE

PULLER.

THERE ARE NUMEROUS OPTIONS FOR "BACK SHAPE SPEC":

1 = FLAT BACK SURFACE

0 = FIND BEST RADIUS OF SPHERICAL CURVATURE

>2 = A SPECIFIED RADIUS OF SPHERICAL CURVATURE (I.E. 96)

0.1 to 0.99 = EDGE TAPER = (EDGE THICKNESS)/(BLANK THICKNESS)

A SPHERICAL BACK WITH A RADIUS OF CURVATURE APPROXIMATELY

EQUAL TO THE FRONT RADIUS OF CURVATURE (DIAMETER x 2f) WILL

FLEX TO AN EXCELLENT PARABOLA.  ALTHOUGH A FLAT BACK ALSO

FLEXES TO A GOOD PARABOLA FOR f6 AND SLOWER, THE SPHERICAL

BACK WILL BE MUCH BETTER FOR FAST MIRRORS.  FOR VERY FAST

MIRRORS THE BACK RADIUS SHOULD BE EVEN LESS.

"TAPER EXPONENT" GOVERNS THE SHAPE OF THE BACK SURFACE TAPER:

0 = A SPHERICAL BACK

1 = LINEAR (A SHALLOW CONE).

A TAPER EXPONENT ABOVE 1 IS CONVEX.  2.0 GIVES A CONVEX

PARABOLA WHICH IS VERY CLOSE TO A SPHERICAL BACK.  0 OR 2

BOTH GIVE ABOUT THE SAME IMAGE QUALITY AND WORK WELL FOR

FAST MIRRORS.  BUT 0 (SPHERICAL) IS FAR MORE PRACTICAL TO

GRIND.

A TAPER EXPONENT BETWEEN 0 AND 1 WILL GIVE A BACK SHAPE

WHICH IS CHARACTERIZED BY A CROSS-SECTION LINE WHICH DIPS

INWARD FROM A LINEAR SLOPE.  0.5 WORKS WELL FOR CENTER

PULLERS.

TAPER EXPONENT IS IGNORED WHEN A FLAT BACK IS SPECIFIED.

FOR ALL BUT THE SPHERICAL BACK, THE EXACT FORMULA FOR BACK

CURVATURE IS Y=K*RADIUS^N.  WHERE K IS COMPUTED TO PRODUCE

THE DESIRED EDGE THICKNESS AND N IS THE TAPER EXPONENT.

"FLEX" DETERMINES IF THE MIRROR WILL BE FLEXED.

FLEX=1 WILL FIND THE BEST FLEX TENSION.

FLEX=0 WILL ANALYZE AN UN-FLEXED SPHERICAL MIRROR.

OUTPUT:

MIRROR IMAGE EVALUATION:

THE PROGRAM COMPUTES SPOT RATIO WHICH IS THE RATIO OF THE

WEIGHTED AVERAGE SPOT DIAMETER DIVIDED BY THE AIRY DISC

DIAMETER.  A SPOT=1.0 CORRESPONDS APPROXIMATELY TO A QUARTER

WAVE WAVEFRONT ERROR, WHICH IS RAYLEIGH'S CRITERIA FOR

DEFRACTION LIMITED OPTICS.

IN EXCELLENT SEEING, CRITICAL OBSERVERS CAN DETECT

REDUCTIONS IN ERROR DOWN TO ABOUT HALF OF RAYLEIGH'S

CRITERIA OR SPOT=0.5.  FURTHER REDUCTIONS ARE PROBABLY NOT

VISUALY DETECTABLE.  BUT CONSIDERING THE MANY SOURCES OF

ERROR IN GRINDING AND FLEXING A MIRROR IT'S WORTH SELECTING

A CONFIGURATION WHICH HAS MINIMAL ERROR.

THE NEXT SCREEN ("G" OPTION) ALSO DISPLAYS THE APPROXIMATE

PEAK TO VALLEY WAVEFRONT ERROR, WHICH IS SIMPLY THE SPOT

RATIO DIVIDED BY FOUR.

THE GRAPHICAL SCREEN DISPLAYS A GRAPH OF TRANSVERSE ERROR

(EFFECTIVELY THE RADIUS OF THE STAR IMAGE).  A PERFECT

PARABOLA WOULD PLOT AS A VERTICAL STRAIGHT LINE CENTERED AT

ZERO.  TRY A 6" f8.1 WITH FLEX=0 (NO FLEX) TO SEE HOW BAD A

QUARTER WAVE MIRROR LOOKS.  THEN CHANGE THE ENTRY ON THE

FLEX LINE TO 1 TO FLEX IT.

WHEN EVALUATING THE GRAPH, KEEP IN MIND THAT THE RAYS NEAR

THE TOP HAVE A GREATER EFFECT ON THE IMAGE BECAUSE THEY

CORRESPOND TO THE OUTER ZONES OF THE MIRROR.  THE INNER RAYS

REPRESENT LESS MIRROR AREA AND ARE LESS CRITICAL.  THE

PROGRAM PROPERLY WEIGHTS THE OUTER RAYS IN FINDING BEST

TENSION AND BEST BETA.  THE PROGRAM DOES NOT EVALUATE THE

PORTION OF THE IMAGE IN THE SECONDARY SHADOW.

THE GRAPHIC SCREEN MAY ALSO BE PLOTTED ON HP/GL COMPATIBLE

PRINTERS BY PRESSING "P".

IF YOUR PRINTER DOESN'T WORK WITH "P", YOU CAN STILL PRINT

THE TEXT DATA BY PRESSING "T".  WITH THIS OPTION YOU CAN

PRINT ABOUT 6 MIRRORS ON A SINGLE PAGE.  THE PAGE WILL EJECT

WHEN FILLED, OR WHEN YOU EXIT THE PROGRAM.

THE OUPUT DATA INCLUDES "PSI", WHICH IS THE TENSILE LOAD ON

THE PULLER.  HIGH DENSITY SPONGE RUBBER FAILS AT A TENSILE

LOAD OF 50 TO 100 PSI.  IF YOUR PSI EXCEEDS 10, YOU SHOULD

REDUCE THE THICKNESS OF THE MIRROR.

OTHER OUTPUTS INCLUDE:

PULLER AND PUSHER PLATE MINIMUM THICKNESS FOR POLYCARBONATE.

FOR ALUMINUM, MULTIPLY THE POLYCABONATE THICKNESS BY 0.35.

THE FOLLOWING THREE OUTPUTS ARE TOLERANCES WHICH APPLY TO

ASTIGMATISM.  ALL OF THESE TOLERANCES INCREASE WITH f RATIO

CUBED, SO AN f6 HAS 3.4X BROADER TOLERANCES THAN AN f4.

IN OTHER WORDS, "SPEED KILLS".

PUSHER UNIFORMITY (%) MEANS HOW UNIFORM THE PERIMETER

PUSHER PRESSURE MUST FOR NO VISIBLE ASTIGMATISM.

PUSHER UNIFORMITY % SHOULD ALSO BE APPLIED TO MIRROR

ROUNDNESS AND TO PULLER AND PUSHER RADIAL CENTERING.

MAX MIRROR WEDGE (INCHES) FOR NO VISIBLE ASTIGMATISM.

MAX GRAVITY EFFECT.  THIS IS THE EFFECT OF CHANGING

ALTITUDE.  100% GRAVITY EFFECT WOULD JUST REACH THE

THRESHOLD OF VISIBLE ASTIGMATISM AT ZERO DEGREES ALTITUDE.

GRAVITY EFFECT IS FACTORED BY THE COSINE OF ALTITUDE.

THUS THERE IS ZERO ATIGMATIC GRAVITY EFFECT AT ZENITH.

WHILE IN THE GRAPHIC SCREEN, TRY THE "W" OPTION TO SEE THE

EFFECT OF CHANGING THE FLEX TENSION.  FOR MODERATE SPEED

MIRRORS, A CHANGE OF TEN PERCENT OR MORE FROM OPTIMUM IS

VISUALY ACCEPTABLE.  FOCUS WOULD SHIFT SLIGHTLY, BUT THE

PROGRAM AUTOMATICALLY ANALYZES THE IMAGE AT BEST FOCUS.

SAVING FILES:

YOU CAN ALSO SAVE THE NINE-LINE MIRROR INPUT SPECIFICATION

WITH THE "S" OPTION AND RETRIEVE IT LATER WITH THE "L"

(LOAD) OPTION.  USE THE STANDARD EIGHT CHARACTER RULES FOR

NAMING DOS FILES.  FOR EXAMPLE YOU MIGHT NAME A 10" f6

MIRROR MIRROR FILE "10F6".  THE FILE WILL BE SAVED IN THE

CURRENT DIRECTORY.  FILES ARE AUTOMATICALLY SAVED WITH THE

.FLX SUFFIX.

THE "B" OPTION PRINTS A TABLE OF THE BACK SURFACE TAPER.

THIS COULD BE USED TO PROGRAM A DIAMOND LATHE.  NOTE THAT

FOR CENTER PULLS WITH A TAPER EXPONENT BETWEEN 0 AND 1,

THE BACK IS FLAT INSIDE THE RADIUS DEFINED BY BETA2.

OUTSIDE OF BETA2 THERE IS A STEP TO THE CURVE.  IT IS

RECOMMENDED THAT THIS STEP BE RADIUSSED (WITH A RADIUS

APPROXIMATELY EQUAL TO THE STEP HEIGHT) WHEN THE BACK

SURFACE IS CUT.

EVALUATING MIRROR TESTS:

THE ACCOMPANYING PROGRAM, ZONE.EXE, PRESENTS MIRROR ZONE

TEST RESULTS IN THE SAME FORMAT AS FLEX.EXE.  THIS IS

CONVENIENT TO COMPARE ACTUAL AND THEORETICAL RESULTS ON

FLEXED MIRRORS HOWEVER THIS PROGRAM MAY BE USED FOR ZONE

TESTS ON ANY MIRROR.

ZONE.EXE CAN ALSO EVALUATE SPHERICAL MIRRORS.  THIS

PROVIDES A QUALITATIVE EVALUATION OF YOUR SPHERE PRIOR TO

FLEXING.

TWO SAMPLE ZONE FILES ARE INCLUDED:

8SS.ZON - A FICTICIOUS 8" f8.9 SPHERE.

86M25.ZON - THE FINAL ZONE TEST ON MY 8" f6 FLEXED MIRROR.